SIMATS ENGINEERING
DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING
CSA12 – COMPUTER ARCHITECTURE
ASSESSMENT TOOL 2 – PSEUDOCODE WRITING TASK

Student Name: Sriram Venkatesan
Register No.: 192372116

This assessment focuses on constructing efficient I/O communication mechanisms using DMA-based transfer, vectored interrupt handling, dynamic I/O mapping, bus arbitration, and hybrid polling–DMA communication. The solutions below are written as structured pseudocode and include initialization, control flow, completion handling, error handling, and efficiency considerations, as required by the assessment. fileciteturn5file0L16-L32

Q1. DMA-Based Data Transfer Algorithm

A real-time embedded system receives large blocks of data continuously from a high-speed I/O device. Direct CPU copying would consume processor cycles and can introduce unnecessary latency. Therefore, a DMA controller is used to transfer data directly from the I/O device to main memory. The CPU only performs initialization, configuration, and completion/error handling.

Objectives and Inputs

Source address: address of the I/O device data register or source buffer.

Destination address: starting address of the main-memory buffer.

Transfer size: number of bytes/words to transfer.

DMA channel: selected DMA controller channel.

Transfer direction: I/O device → main memory.

Interrupt sources: DMA completion and DMA error.

Buffer status: available/occupied state to prevent memory overwrite.

Pseudocode

ALGORITHM DMA_Data_Transfer
INPUT: source_address, destination_address, transfer_size
OUTPUT: transfer_status

BEGIN
    Disable_DMA_Channel(DMA_CHANNEL)

    DMA.source  ← source_address
    DMA.dest    ← destination_address
    DMA.count   ← transfer_size
    DMA.direction ← IO_TO_MEMORY
    DMA.mode    ← BLOCK_TRANSFER
    DMA.interrupt_on_complete ← TRUE
    DMA.interrupt_on_error    ← TRUE

    Clear_DMA_Status(DMA_CHANNEL)

    IF NOT Is_DMA_Configuration_Valid(DMA) THEN
        Report_Error("Invalid DMA configuration")
        RETURN ERROR
    END IF

    Enable_DMA_Channel(DMA_CHANNEL)
    Start_DMA(DMA_CHANNEL)

    WHILE DMA.busy = TRUE DO
        Perform_Other_CPU_Tasks()
    END WHILE

    IF DMA.error_flag = TRUE THEN
        Handle_DMA_Error()
        transfer_status ← ERROR
    ELSE IF DMA.complete_flag = TRUE THEN
        Acknowledge_DMA_Completion()
        Update_Buffer_Status(destination_address, READY)
        transfer_status ← SUCCESS
    END IF

    RETURN transfer_status
END

Detailed Working

Disable the DMA channel before programming it so that old configuration values cannot cause an unintended transfer.

Load the source address, destination address, transfer count, transfer direction, and transfer mode.

Enable completion and error interrupts.

Validate the configuration, including address range, transfer size, alignment, and buffer availability.

Enable the DMA channel and start the transfer.

The DMA controller requests bus access and transfers the data without CPU copying.

The CPU performs other useful work while DMA is busy.

When the transfer completes, the DMA controller raises a completion interrupt.

If an error occurs, an error interrupt is generated and the error handler records the condition and safely stops/reinitializes the channel.

After successful completion, the destination buffer is marked ready for processing.

DMA Completion and Error ISR

ISR DMA_Interrupt_Handler
BEGIN
    status ← Read_DMA_Status()

    IF status.error_flag = TRUE THEN
        Disable_DMA_Channel()
        Save_Error_Code(status.error_code)
        Clear_DMA_Error()
        Notify_Error_Handler()
        RETURN
    END IF

    IF status.complete_flag = TRUE THEN
        Clear_DMA_Completion_Flag()
        Mark_Current_Buffer_Ready()
        Notify_Data_Processing_Task()
    END IF
END

Why DMA Minimizes CPU Involvement

For a large transfer, the CPU should not execute a load from the device followed by a store to memory for every word. DMA performs these repeated transfers in hardware. The CPU instead handles only configuration and completion/error events. For continuous high-speed streams, double buffering or circular buffering can be added so that DMA fills one buffer while the CPU processes another.

Final Result

The algorithm satisfies the required sequence of DMA initialization, source/destination configuration, transfer-size setup, transfer initiation, and completion/error handling. It provides direct I/O-to-memory transfer and minimizes CPU involvement. The assessment specifically requires these DMA stages and interrupt cases. fileciteturn5file0L33-L38

Q2. Prioritized Vectored Interrupt Handling

Multiple I/O devices can generate interrupts with different priority levels. A vectored interrupt controller is used so that the interrupt source can be identified quickly and the CPU can directly dispatch the corresponding interrupt service routine (ISR). High-priority requests must be serviced before lower-priority work, while lower-priority requests must remain pending rather than being lost.

Inputs and Data Structures

Interrupt request register (IRR): records pending interrupt sources.

Priority table: stores the priority of each device.

Vector table: maps each device to its ISR.

Interrupt mask: enables/disables selected interrupt sources.

Current interrupt priority: indicates the priority currently being serviced.

Pending queue: retains lower-priority requests.

Pseudocode

ALGORITHM Prioritized_Vectored_Interrupt_Handler
INPUT: interrupt_request
OUTPUT: serviced_interrupt

BEGIN
    IF interrupt_request = NONE THEN
        RETURN NO_INTERRUPT
    END IF

    pending_list ← Read_Pending_Interrupts()

    WHILE pending_list is not EMPTY DO
        selected ← Select_Highest_Priority(pending_list)

        IF Priority(selected) < Current_Priority() THEN
            Defer(selected)
            Remove(selected, pending_list)
            CONTINUE
        END IF

        vector ← Vector_Table[selected]
        Acknowledge_Interrupt(selected)

        Save_Minimum_CPU_Context()
        Set_Current_Priority(Priority(selected))

        Dispatch_To_ISR(vector)

        Restore_CPU_Context()
        Restore_Previous_Priority()

        Remove(selected, pending_list)
        pending_list ← Read_Pending_Interrupts()
    END WHILE

    RETURN SUCCESS
END

ISR Dispatch Logic

PROCEDURE Dispatch_To_ISR(vector)
BEGIN
    SWITCH vector
        CASE VECTOR_SENSOR:
            ISR_Sensor()
        CASE VECTOR_TIMER:
            ISR_Timer()
        CASE VECTOR_STORAGE:
            ISR_Storage()
        CASE VECTOR_COMMUNICATION:
            ISR_Communication()
        CASE VECTOR_ERROR:
            ISR_System_Error()
        DEFAULT:
            ISR_Unknown_Interrupt()
    END SWITCH
END

Nested Interrupt Handling

If a low-priority ISR is running and a higher-priority interrupt arrives, the higher-priority request can preempt the current ISR. The current CPU context is saved, the high-priority vector is dispatched, and the processor returns to the interrupted ISR after completion. This avoids delaying urgent real-time events.

Avoiding Starvation

Use fixed priorities for urgent devices, but retain lower-priority requests in a pending queue.

Keep ISRs short and defer lengthy processing.

Use priority-aware preemption for critical interrupts.

Use interrupt masking only for short critical sections.

Use interrupt coalescing for very high-rate devices where latency permits.

Optionally use aging: gradually increase the effective priority of a request that has waited too long.

Example Priority Table

| Device | Priority | Vector | Typical Action |
| --- | --- | --- | --- |
| Emergency/error device | 1 – Highest | VEC_ERROR | Immediate fault handling |
| Real-time sensor | 2 | VEC_SENSOR | Capture time-critical data |
| Timer | 3 | VEC_TIMER | Schedule real-time work |
| Storage | 4 | VEC_STORAGE | Process completed I/O |
| Communication | 5 | VEC_COMMUNICATION | Process packets |

This mechanism directly addresses the requirement to identify vectored interrupts, determine the interrupting device, dispatch the appropriate ISR, efficiently service high-priority requests, and correctly handle lower-priority requests. fileciteturn5file0L39-L43

Q3. Dynamic I/O Mapping Selection

Different I/O devices have different latency, bandwidth, and data-transfer requirements. The algorithm must dynamically choose between memory-mapped I/O (MMIO) and I/O-mapped I/O (PMIO/I/O-mapped I/O) according to device requirements.

Decision Criteria

Latency requirement: strict real-time devices should receive the lowest-overhead mapping available on the target processor.

Bandwidth requirement: high-bandwidth devices should use a mapping that integrates efficiently with the processor and bus architecture.

Access frequency: devices accessed very frequently benefit from low-overhead register access.

Address-space availability: MMIO consumes part of the memory address space.

Processor support: the final choice must be supported efficiently by the CPU architecture.

Device characteristics: legacy devices may naturally use a dedicated I/O address space.

Pseudocode

ALGORITHM Select_IO_Mapping
INPUT:
    device_type
    latency_requirement
    bandwidth_requirement
    access_frequency
    address_space_available
    processor_supports_io_mapped

OUTPUT:
    selected_mapping

BEGIN
    IF device_type = "HIGH_SPEED" THEN
        IF latency_requirement = "STRICT" AND
           address_space_available = TRUE THEN
            selected_mapping ← MEMORY_MAPPED_IO
        ELSE
            selected_mapping ← MEMORY_MAPPED_IO
        END IF

    ELSE IF latency_requirement = "STRICT" THEN
        IF address_space_available = TRUE THEN
            selected_mapping ← MEMORY_MAPPED_IO
        ELSE IF processor_supports_io_mapped = TRUE THEN
            selected_mapping ← IO_MAPPED_IO
        ELSE
            selected_mapping ← MEMORY_MAPPED_IO
        END IF

    ELSE IF bandwidth_requirement = "HIGH" AND
            access_frequency = "HIGH" THEN
        selected_mapping ← MEMORY_MAPPED_IO

    ELSE IF processor_supports_io_mapped = TRUE AND
            bandwidth_requirement = "LOW" THEN
        selected_mapping ← IO_MAPPED_IO

    ELSE
        selected_mapping ← MEMORY_MAPPED_IO
    END IF

    RETURN selected_mapping
END

Mapping Characteristics

| Requirement | Preferred Technique | Reason |
| --- | --- | --- |
| Very low latency, frequent register access | Memory-mapped I/O | Uses normal load/store operations and integrates naturally with CPU address space |
| High-bandwidth peripheral control | Memory-mapped I/O | Convenient for frequent register access and modern bus systems |
| Legacy/special-purpose device with dedicated I/O space | I/O-mapped I/O | Separates device ports from normal memory address space |
| Limited memory address-space availability | I/O-mapped I/O, if supported | Preserves memory address range |
| Modern embedded/SoC peripheral registers | Memory-mapped I/O | Common and simple programming model |

Latency and Bandwidth Evaluation

The selection should not assume that one technique is universally faster on every processor. The algorithm evaluates the actual constraints and then uses the architecture-supported option with the lowest effective access overhead. For modern embedded systems, MMIO is often the practical default because peripheral registers are naturally integrated into the processor's address map. I/O-mapped I/O remains useful when the processor provides a separate I/O space and the device or system design benefits from it.

The assessment requires the algorithm to evaluate latency and bandwidth requirements and select the mapping technique that gives efficient communication. fileciteturn5file0L44-L47

Q4. Fair Bus Arbitration for PCIe, USB and Thunderbolt

Several devices may simultaneously request access to high-speed communication resources such as PCIe, USB, and Thunderbolt. A bus arbitration mechanism must consider priority, bandwidth demand, interface availability, and fairness. The goal is to maximize resource utilization without allowing a high-bandwidth device to permanently starve other devices.

Inputs

Device ID.

Interface type: PCIe, USB, or Thunderbolt.

Priority level.

Requested bandwidth.

Available interface bandwidth.

Current allocation and recent service history.

Waiting time, used to prevent starvation.

Pseudocode

ALGORITHM Fair_Bus_Arbitration
INPUT: request_queue, interface_status
OUTPUT: granted_requests

BEGIN
    granted_requests ← EMPTY

    WHILE request_queue is not EMPTY DO

        FOR each request IN request_queue DO
            request.wait_time ← Current_Time - request.arrival_time

            IF request.wait_time > STARVATION_LIMIT THEN
                request.effective_priority ← MAX_PRIORITY
            ELSE
                request.effective_priority ←
                    request.base_priority + Aging_Factor(request.wait_time)
            END IF
        END FOR

        candidate_list ← Sort_By(
            effective_priority DESCENDING,
            wait_time DESCENDING,
            bandwidth_efficiency DESCENDING
        )

        granted ← FALSE

        FOR each request IN candidate_list DO
            interface ← request.interface

            IF Interface_Available(interface) AND
               Bandwidth_Available(interface, request.bandwidth) THEN

                Allocate(interface, request.device)
                Reserve_Bandwidth(interface, request.bandwidth)
                Update_Service_History(request.device)

                Add(granted_requests, request)
                Remove(request, request_queue)

                granted ← TRUE
                BREAK
            END IF
        END FOR

        IF granted = FALSE THEN
            Schedule_Next_Arbitration_Round()
            BREAK
        END IF
    END WHILE

    RETURN granted_requests
END

Weighted Fairness

A weighted round-robin policy can be used when devices have different bandwidth requirements. For example, a Thunderbolt device may receive more service opportunities than a low-speed USB device, but the USB device must still receive periodic service. Priority determines urgency, while weighting and aging maintain fairness.

| Interface | Typical Role | Arbitration Consideration |
| --- | --- | --- |
| PCIe | High-speed internal peripherals/storage | High bandwidth; allocate according to link capacity and device priority |
| USB | Multiple peripherals | Fair scheduling among devices sharing a host controller |
| Thunderbolt | Very high-speed external devices | High bandwidth allocation with starvation protection |

Example

Suppose Thunderbolt-1 requests 8 GB/s, Thunderbolt-2 requests 6 GB/s, USB-1 requests 0.5 GB/s, and USB-2 requests 0.2 GB/s. The arbiter should not simply select the largest request repeatedly. It should consider interface availability, priority, and waiting time. If USB-1 has waited beyond the starvation threshold, its effective priority is increased so that it receives service in the next feasible arbitration round.

Final Result

The algorithm provides priority-aware allocation while using aging and waiting time to prevent starvation. It also checks actual interface availability and bandwidth before granting a request. This directly satisfies the requirement to consider priority, bandwidth, interface availability, and fairness. fileciteturn5file0L48-L52

Q5. Hybrid Polling–DMA Communication Mechanism

The final problem contains both low-speed devices, such as simple sensors, and high-speed devices, such as data acquisition units and storage devices. Polling is appropriate for low-rate devices because interrupt/DMA setup overhead may not be justified. High-speed devices require DMA and interrupt-driven completion notification so that the CPU does not repeatedly move large amounts of data.

Device Classification

| Device Type | Typical Example | I/O Technique | CPU Involvement |
| --- | --- | --- | --- |
| LOW_SPEED | Simple temperature/status sensor | Polling | Low but periodic |
| HIGH_SPEED | Data acquisition unit | DMA + interrupts | Very low during bulk transfer |
| HIGH_SPEED | Storage device | DMA + interrupts | Low; CPU handles commands/completions |

Pseudocode

ALGORITHM Hybrid_IO_Communication
INPUT: device_list
OUTPUT: communication_status

BEGIN
    Initialize_CPU()
    Initialize_DMA_Controller()
    Initialize_Interrupt_Controller()

    FOR each device IN device_list DO

        IF Device_Speed(device) = LOW_SPEED THEN
            Configure_Polling(device)
            device.mode ← POLLING

        ELSE IF Device_Speed(device) = HIGH_SPEED THEN
            Configure_DMA(device)
            Configure_Completion_Interrupt(device)
            Configure_Error_Interrupt(device)
            device.mode ← DMA_INTERRUPT

        ELSE
            Mark_Device_As_Unsupported(device)
        END IF
    END FOR

    WHILE System_Running = TRUE DO

        FOR each device IN device_list DO

            IF device.mode = POLLING THEN
                status ← Read_Device_Status(device)

                IF status.data_ready = TRUE THEN
                    data ← Read_Small_Data(device)
                    Store_Data(data)
                END IF

                IF status.error = TRUE THEN
                    Handle_Polling_Device_Error(device)
                END IF

            ELSE IF device.mode = DMA_INTERRUPT THEN
                IF DMA_Request_Required(device) THEN
                    Prepare_DMA_Descriptor(device)
                    Start_DMA(device)
                END IF
            END IF

        END FOR

        Process_Pending_Interrupts()

        Perform_Background_Tasks()
    END WHILE

    RETURN SUCCESS
END

High-Speed DMA Interrupt Handler

ISR High_Speed_Device_Handler
BEGIN
    status ← Read_Device_And_DMA_Status()

    IF status.error = TRUE THEN
        Stop_DMA()
        Record_Error(status.error_code)
        Recover_Or_Reset_Device()
        Notify_Error_Task()
        RETURN
    END IF

    IF status.transfer_complete = TRUE THEN
        Clear_Completion_Flag()
        Mark_Buffer_Ready()
        Queue_Data_For_Processing()
    END IF
END

Low-Speed Polling Routine

PROCEDURE Poll_Low_Speed_Devices
BEGIN
    FOR each low_speed_device DO
        status ← Read_Status_Register(low_speed_device)

        IF status.ready = TRUE THEN
            data ← Read_Data_Register(low_speed_device)
            Store_Data(data)
        END IF

        IF status.error = TRUE THEN
            Handle_Device_Error(low_speed_device)
        END IF
    END FOR
END

Why Hybrid I/O Is Efficient

Low-speed sensors are checked periodically without generating frequent interrupts.

The CPU does not waste DMA setup time on very small transfers.

High-speed devices use DMA so large blocks move directly to memory.

Interrupts notify the CPU only when high-speed transfers complete or fail.

Circular or double buffers can be used for continuous high-speed streams.

The CPU remains available for application processing instead of copying every byte.

Example Scheduling

A simple sensor may be polled every 10 ms because its value changes slowly. A high-speed acquisition device may continuously fill a DMA buffer. When the buffer becomes full, the DMA controller generates an interrupt, the CPU marks the buffer ready, and processing occurs while the next buffer is filled. This creates an overlap between acquisition and processing.

Final Result

The hybrid algorithm first identifies the device class, selects polling for low-speed devices, and configures DMA plus interrupt-driven communication for high-speed devices. It handles both completion and error conditions while minimizing CPU involvement. This matches the required hybrid polling–DMA design specified in the assessment. fileciteturn5file0L53-L58

Overall Conclusion

The five pseudocode solutions demonstrate how different I/O requirements require different control mechanisms. DMA is appropriate for large high-speed transfers because it moves data with minimal CPU intervention. Vectored and prioritized interrupts provide rapid response to urgent devices while preserving lower-priority requests. Dynamic I/O mapping allows the system to select a suitable register-access mechanism according to latency, bandwidth, address-space and processor constraints.

Fair bus arbitration combines priority with bandwidth awareness and starvation prevention so that PCIe, USB and Thunderbolt resources are used efficiently. Finally, hybrid I/O combines the simplicity of polling for low-speed devices with the efficiency of DMA and interrupts for high-speed devices.

Together, these algorithms address the assessment requirements for DMA transfer, interrupt prioritization, dynamic I/O mapping, bus arbitration, and hybrid polling–DMA communication, while emphasizing CPU utilization, latency, throughput, bandwidth, resource allocation, and overall system suitability. fileciteturn5file0L22-L32